LEDTask™ Super High Output LTR120024WP-HO-  WET LOCATION  24VDC  Linear Ribbon Installation Instructions using LTCWP and LTW- WP series connectors

*MAKE SURE LEDTask AMPERAGE DOESN’T EXCEED POWER SUPPLY*

****ONLY USE 24 VOLT POWER SUPPLIES****

Make all connections and test system before installing

MAX RUN  ON A SINGLE POWER LEAD FOR =  16.4 feet.

- Cutting the LED ribbon to desired length: Using scissors the LED ribbon can be cut on the clearly marked cutting line which is located every 6 LEDS, (FIG 1)except where the solder drops are located on the ribbon. The solder is located every 19-1/2 inches on the ribbon. Making a cut anywhere else on the ribbon except on the cutting line will cause damage to the circuit and the ribbon will not operate properly. Where the ribbon is cut,  is the termination point.  Use enclosed liquid silicone and ribbon end cap to seal ribbon. NOTE: cut portion of ribbon cannot be used.

- Connecting ribbon to power supply. Using the water proof plugs(FIG 3), seat them tightly to ensure a good seal. Verify the polarity is correct.  The  LTW and LTC wet location connectors are keyed to have a proper electrical orientation and cannot be put in upside down.    Test connection for continuity.

- Connecting ribbon sections together. (one meter lengths only) Using the LTCWP-12(24”, 36” or 60”)   Verify polarity is correct(FIG 4). Test connection for continuity.

- Make sure to verify polarity. Verify continuity before installing the ribbon(FIG 6). The and LTW and LTC wet location connectors are keyed to have a proper electrical orientation and cannot be put in upside down. Use included mounting clips(FIG 5) and/or silicone adhesive(by others). Painters tape can be used to hold ribbon in place to allow ample cure time for adhesives.