LEDTask™ 12VDC LTR300WP-SO, LTR60WP-SO, LTR300-WP AND LTR60WP

Wet Location LED Linear Ribbon Installation Instructions

MAKE SURE LEDTask AMPERAGE DOESN’T EXCEED POWER SUPPLY*

Make all connections and test system before installing

MAX RUNS- LTR60WP AND LTR300WP = 16.4 FT – LTR60WP-SO AND LTR300WP-SO = 32.8FT.

- Cutting the LED ribbon to desired length: Using scissors the LED ribbon can be cut on the clearly marked cutting line which is located every 3 LEDS (FIG 1) except where the solder drops are located on the ribbon. The solder is located every 19-1/2 inches on the ribbon. Making a cut on the solder or anywhere else on the ribbon except on the cutting line will cause damage to the ribbon and it will not operate.  NOTE: The piece of ribbon left over from your cut CANNOT be used – keep this in mind when designing the layout.

- After the cut is made: Fill the end cap with the silicone sealant provided and slide over ribbon (Fig 2). Smooth overfill along the edge of end cap to ensure watertight seal (allow to dry overnight).

- Connecting multiple reels together: plug female connector at end of reel into male connector at beginning of the 2nd reel. Follow these steps when using LTCWP ribbon connectors. (Fig 3)

- Wiring Ribbon to Power Supply: Plug the LTW-24-WP series waterproof female connector into the male connector affixed to the start of the reel. Wire back to the power supply (Fig 4). NOTE: Wire with white stripe on female connector is ‘positive’. Power supplies must be installed in waterproof listed enclosures or a watertight structure when installed outside; excluding LTHM50-DIM, LTHM100-DIM and LTHM300-DIM, which are listed for wet location.

- Mounting ribbon to surface: Use the mounting clips and screws provided to attach ribbon to surface (Fig 5).

Four clips per meter are included. NOTE: Avoid mounting the clips directly over an LED.

- Securing connections: Use additional clips to secure connectors located at the end of the run to the mounting surface

(Fig 6).  THIS PRODUCT IS UL/ETL LISTED ‘WET LOCATION’ AND IS NOT INTENDED TO BE SUBMERGED DIRECTLY IN WATER.